bp Energy Company of Trinidad and Tobago

Job Safety Analysis

| Location /System |  | E002-07 |  | Date | 19th July 2007 |  | JSA # | Site Visited: Yes No | # Permit # Not Applicable |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Job/Task Description |  | Diving within Guayaguayare Bay |  |  |  |  |  |  |  |  |
| Prepared by : Coastal Dynamics |  |  |  |  |  |  |  |  |  |  |
| Has consideration been given to weather conditions Yes No |  |  | Are written job procedures discussed and attached Yes No |  |  | Are there SIMOPS : Yes No |  |  |  | Emergency Response Services Notified : Yes No NA |

Hazard Effect (HE)x Probability(P) =Risk(R) *R  Signifies residual risk after control measures in place

| Sequence of basic job steps | Hazards Identified Use Checklist as a guide to be specific | Hazard effect (type of injury / damge or environmental effect) | HE | P | R | Corrective Measures / Recommendations | HE | P* | R* | ALARP Y/N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Driving vehicle from Port of Spain to bpTT’s ASCO Base in Point Galeota | Mechanical problems, accidental vehicle conditions | Broken limbs, death |  |  |  | Vehicle kept in good working order. Seatbelts to be worn at all times while vehicle is in motion. Driver must be competent. No talking on cellular phones by driver. No horseplay. Driver should avoid distractions such as changing radio stations while driving. Speed limit must be obeyed. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Toolbox meeting |  |  |  |  |  | Role of each member to be clearly explained. Environmental and water conditions to be assessed. If divers and/or boatman do not feel comfortable continuing with trip, it is to be cancelled. Divers to inform trip leader if feeling unwell and sit out dives if deemed necessary. Dive safety officer to be identified |  |  |  |  |
| Loading and Accessing the boat | Possible accidental slippage on wet surfaces, Heavy equipment, unstable boat | Back injury, |  |  |  | Follow proper lifting techniques. Ensure boat is not rocking or overly unstable. Engine must be switched off. If needed access should be via ladder. |  |  |  |  |
| Driving of boat to and from dive site | Possible falling over the side of the boat, collision with rocky outcrops, tilting of boat | Drowning, capsizing of boat, damaged hull |  |  |  | Boat operator must be qualified and familiar with Guayaguayare Bay. Boat captain to ensure boat is properly balanced prior to leaving. Dive tanks must be secured from rolling. All persons on board must wear life vest. A working radio to be kept on board |  |  |  |  |
| Divers entering the water | Accidental slippage, | Injury to body, |  |  |  | Divers with training beyond PADI Advanced Openwater. Ensure engine is switched off. Headcount to be conducted. |  |  |  |  |
| Diving underwater | Possibility of other vessels encroaching dive area. Uncontrolled ascent | Damage to dive equipment. Nitrogen coming out of solution causing ‘bends’ |  |  |  | Dive flag to be deployed. Certified and experience divers only. Crew members to immediately contact recompression chamber facilities using the numbers on sheet attached to JSA. Diving operations to be immediately discontinued and all divers to return to boat. Divers should not be given pain killers. If diver is conscious, diver to be given as much water as can tolerate. Treatment to be carried out under sunshade. |  |  |  |  |
| Crew staying on board while diving taking place | Possible dehydration, possible target for engine theft | Sun stroke, boat unable to move |  |  |  | Crew members should regularly apply sunblock and stay hydrated. Crew should try to stay under sunshade as much as possible. All crew members should be vigilant and aware of surroundings. Radio should be kept in an easily accessible location. Coast Guard to be notified of diving operations. |  |  |  |  |
| Divers getting back on boat | Possible contact with engine’s propellers, slips trips and falls, leaving diver behind | Cuts about body, possible dismemberment, injury to body. |  |  |  | Boat’s engine must be switched off, divers to use ladder. BCD, regulators, fins to be handed to top vessel personnel before climbing ladder. Headcount to be conducted prior to heading back to shore. Divers to carry whistles or signaling devices. |  |  |  |  |
| Unpacking of boat and conclusion of field trip | Heavy equipment such as dive tanks, exhaustion | Possible back injury, fainting, falling ill |  |  |  | Proper lifting procedures to be followed, no diver should engage in overly strenuous activity. All crew members, especially divers to drink enough water. Divers to monitor their bodies and symptoms for next 24 hours, notifying relevant person if necessary. Divers instructed to not take very hot showers and engage in strenuous activity 18 -24 hours after diving. |  |  |  |  |

HAZARD CHECKLIST : - Falls from heights              - Falling objects         Collision      Access/Egress,      Moving Machinery       Noise    Chemical exposure        Illumination        Flammable Materials

Flaring / hydrocarbon gas release      Oil/ Chemical         Competence of personnel       Confine Space        Noise       Manual Handling         Corrosive Material       Naked Flames

Electricity         Liquid or Gas under Pressure

Has tool box meeting been held : Yes       No

| Signatures of Job Crew |  |  | Date : |
| --- | --- | --- | --- |

| Performing Authority | Date : | Area Authority : | Date : |
| --- | --- | --- | --- |